Data Validation Summary Report for Asbestos Data Associated with the Post-Remediation Screening Health Risk Assessment Report for Parcels C, D, F, G and H, Nevada Environmental Response Trust (NERT)

Prepared for:

Nevada Division of Environmental Protection 
Bureau of Corrective Actions 
2030 E. Flamingo Road, Suite 230 
Las Vegas, Nevada 89119-0818

February 10, 2014

Prepared by:

Neptune and Company, Inc.
1435 Garrison St., Suite 110
Lakewood, CO 80215
Point of Contact: Rebecca Shircliff

Table of Contents

1.	Introduction	1

1.1.	PARCCS Criteria	3

1.2.	Basis for Qualifying Data	5

2.	Asbestos via EPA Method 540-R-97-028 with Berman and Kolk (2000) Modifications  (soils)	5

2.1.	Quality Control Results	6

2.1.1.	Blank Samples	6

2.1.2.	Duplicate Sample Results	6

2.2.	Unaddressed Issues	6

2.3.	Summary	8

3.	PARCCS	9

References	9

List of Acronyms and Abbreviations

BMI	Basic Management, Inc.

BSD	Bench Sheet Data

CoC	chain of custody

DVSR	data validation summary report

ED	electron diffraction

EDD	electronic data deliverable

EDXA	energy dispersive X-ray analysis

EPA	Environmental Protection Agency (U.S.)

NAM	non-asbestos mineral

NDEP	Nevada Division of Environmental Protection

NERT	Nevada Environmental Response Trust

PARCCS 	precision, accuracy, representativeness, completeness, comparability and sensitivity

%C 	percent completeness

QA	quality assurance

QC 	quality control

RPD	relative percent difference

S	analytical sensitivity

SOP	standard operating procedure

TEM	transmission electron microscope

# Introduction

This data validation summary report (DVSR) has been prepared by Neptune and Company, Inc. to assess the validity and usability of asbestos data reported by EMSL Analytical, Inc. for the NERT Post-Remediation Screening Health Risk Assessment (HRA) Report for Parcels C, D, F, G and H (NERT, 2013). This asbestos-only DVSR also includes an electronic data deliverable (EDD) that contains all analytical results and any qualifiers applied during this validation.

The laboratory reports from EMSL contained results for soil samples analyzed for asbestos via transmission electron microscopy (TEM). The asbestos soil samples were collected from the Basic Management, Inc. (BMI) Complex site. To determine the amount of releasable asbestos via dust generation, the sampling and analysis U.S. Environmental Protection Agency (EPA) Method 540-R-97-028 (with Berman and Kolk Modifications, 2000) was utilized. Table 1-1 below identifies the 79 soil samples for which asbestos data have been validated for this DVSR. These samples are also listed in Table 4 and D10 of the HRA.

Table 1-1.  NERT Parcels C, D, F, G and H HRA Asbestos Samples

| Sample IDs |  |  |  |
| --- | --- | --- | --- |
| G1-PC-1-1-0.0 | TSB-DR-01-0 | TSB-FR-01-0 | TSB-HJ-04-0 |
| H2-PC-1-1-0.0 | TSB-DR-02-0 | Q3-PF-3-1-0.0 | TSB-HJ-05-0 |
| TSB-CJ-01-0 | TSB-DR-02-0 FD | TSB-FR-03-0 | TSB-HJ-06-0 |
| TSB-CJ-02-0 | TSB-DR-03-0 | TSB-FR-04-0 | TSB-HJ-07-0 |
| TSB-CJ-04-0 | TSB-DR-04E-0 | TSB-FR-04-0-FD | TSB-HJ-08-0 |
| TSB-CJ-05-0 | TSB-DR-04W-0 | TSB-FR-05-0 | TSB-HJ-09-NE-0 |
| TSB-CJ-06-0 | TSB-DR-04W-0-FD | S3-PG-1-1-0.0 | TSB-HJ-10-0 |
| TSB-CJ-07-0 | TSB-DR-05-0 | TSB-GJ-01-0 | TSB-HJ-11-0 |
| TSB-CJ-08-0 | TSB-DR-06-0 | TSB-GJ-02-0 | TSB-HJ-12-0 |
| TSB-CJ-10-0 | P2-PF-1-1-0.0 | TSB-GJ-03-0 | TSB-HJ-13-0 |
| TSB-CJ-11-0 | P3-PF-1-1-0.0 | TSB-GJ-05-0 | TSB-HR-01-0 |
| TSB-CR-01-0 | P3-PF-2-1-0.0 | TSB-GJ-06-0 | TSB-HR-02-0 |
| TSB-CR-01-0 FD | Q2-PF-1-1-0.0 | TSB-GJ-07-0 | TSB-HR-03-0 |
| TSB-CR-04-0 | Q3-PF-1-1-0.0 | TSB-GJ-08-0 | TSB-HR-04-0 |
| TSB-CR-05-0 | TSB-FJ-04-0 | TSB-GR-01-0 | TSB-HR-05-0 |
| TSB-CR-06-0 | P4-PF-1-1-0.0-FD | TSB-GR-01-0-FD | TSB-HR-07 |
| TSB-CR-07-0 | P4-PH-1-1-0.0 | TSB-GR-02-0 | TSB-HR-08 |
| E1-PC-1-1-0.0 | TSB-FJ-05-0-FD | TSB-HJ-01-0 | V5-PH-1-1 |
| F4-PD-1-1-0.0 | TSB-FJ-09-0 | TSB-HJ-02-0 | W4-PH-1-1 |
| TSB-DJ-01-0 | TSB-FJ-10-0 | TSB-HJ-03-0 |  |

The laboratory reports included a summary report and a bench data sheet for each asbestos sample. This summary information included elutriator data, TEM specifications, structure counts, classification and sketches. Note that chain of custody (CoC) documents (except for a select few), TEM images, TEM calibration, electron diffraction (ED) and energy dispersive X-ray analysis (EDXA) results were not included with the laboratory reports. Field duplicates were analyzed for precision and included with the reports. EMSL maintains data for TEM calibration and blank samples, which are typically not included in the lab reports due to the nature of elutriator sampling (i.e., it could take months before enough samples are analyzed to warrant a blank, such as filter lot, method, etc.).

The laboratory reports were validated following 2012 Nevada Division of Environmental Protection (NDEP) guidance for validating asbestos data in soils (NDEP, 2012). Acceptance criteria for the QC samples were based upon the associated analytical method (540-R-97-028) and the modified elutriator method (Berman and Kolk, 2000). In cases where the analytical method did not fully describe the quality assurance (QA) criteria or corrective action, the U.S. Department of Defense’s Quality Systems Manual for Environmental Laboratories, Version 4.2, was used as a guide for acceptance criteria. Professional judgment was also used in some cases to qualify the results.

This DVSR summarizes the QA evaluation of the data according to precision, accuracy, representativeness, completeness, comparability, and sensitivity (PARCCS). This DVSR provides an assessment of the data and identifies potential sources of error, uncertainty, and bias that may affect the overall usability. Data qualifiers and their definitions are provided below.

| Qualifiers |  |
| --- | --- |
| J- | Estimated: The associated numerical value is an estimated quantity with a potentially negative bias. The analyte was detected, but the reported value may not be accurate or precise. The "J-" qualification indicates the data fell outside the QC limits, but the exceedance was not sufficient to cause rejection of the data. |
| J+ | Estimated: The associated numerical value is an estimated quantity with a potentially positive bias. The analyte was detected, but the reported value may not be accurate or precise. The "J+" qualification indicates the data fell outside the QC limits, but the exceedance was not sufficient to cause rejection of the data. |
| J | Estimated: The associated numerical value is an estimated quantity. It is not possible to assess the direction of the potential bias. The analyte was detected, but the reported value may not be accurate or precise. The "J" qualification indicates the data fell outside the QC limits, but the exceedance was not sufficient to cause rejection of the data. |
| UJ | Estimated/nondetected: Analyses were performed for the compound or analyte, but it was not detected. This qualification is used to flag possible false negative results in the case where low bias is indicated by a detect in the field duplicate. |
| R | Rejected: The datum is unusable (the compound or analyte may or may not be present). Use of the "R" qualifier indicates a significant variance from functional guideline acceptance criteria. |

| Qualifier Reason Codes |  |
| --- | --- |
| b | The analyte was detected in the associated laboratory blank |
| fd | Field duplicates did not meet the project control limits |

## PARCCS Criteria

Precision is a measure of the agreement or reproducibility of analytical results under a given set of conditions. It is a quantity that cannot be measured directly but is calculated from structure counts. Precision is expressed as the relative percent difference (RPD):

RPD = Absolute Value of (D1-D2)*2/(D1+D2) * 100

Where D1 and D2 are the reported structure counts for sample and duplicate analyses.

An RPD outside the 50% QC limit (Berman and Kolk, 2000) between the field duplicates indicates imprecision but does not imply accuracy or allow for directional qualification (e.g., J+ or J-). Note that due to the inherent heterogeneity of soil samples and site locations, a high occurrence of RPD exceedances may be observed. However, it is important for field duplicates to be evaluated to assess site (and possibly sampling) variability. For this report, only duplicate field samples were provided (i.e., no laboratory duplicates were analyzed).

Accuracy is a measure of the agreement of an experimental determination and the true value of the parameter being measured. Due to the nature of asbestos analysis, accuracy cannot easily be assessed. There are no set standards or reference materials that can mimic the type of samples collected at naturally occurring sites. The key component for accuracy is the analyst, who is well trained for the identification and analysis of asbestos structures, including proficiency tests generated by accrediting agencies (e.g., NVLAP). The analyst uses tools such as morphology, ED and EDXA to accurately identify the asbestos structure and visually determines its size using scale bars. Note that the equipment, such as the TEM, ED and EDXA, have manufacturer requirements for maintaining its proper use and calibration; these records are typically maintained by the laboratory and must be requested with data packages. For this DVSR, the instrument calibration (e.g., camera, magnification, K-factors, detector resolution, resolvable Mg-Si and Na peaks, spot size measurement) will not be discussed since this information was not provided.

Representativeness is a qualitative parameter that expresses the degree to which the sample data are characteristic of a population. It is evaluated herein by reviewing the QC results of blanks, samples and holding times. Positive detects of compounds in the blank samples identify compounds that may have been introduced into the samples during sample collection, transport, preparation, or analysis. The QC blanks collected and analyzed can include filter lot, field, lab, method, equipment and conditioning filter blanks. The client (i.e., who requests asbestos analysis) determines if field blanks are collected/analyzed, whereas the other blanks are dependent on batch sizes and if contamination is found (e.g., conditioning filters). Holding times and preservation are not established in the EPA Method (540-R-97-028) or Berman and Kolk (2000) modifications; however, the EMSL Elutriator SOP (rev. 2.1, June 2010) recommends samples be shipped on ice and stored at ice temperature if samples are not immediately analyzed. This is to avoid bacterial growth within the samples.

Comparability is a qualitative expression of the confidence with which one data set may be compared with another. In the data validation context, it provides an assessment of the equivalence of the analytical results to data obtained from other analyses. Comparability is also dependent upon other PARCCS criteria because only when precision, accuracy, and representativeness are known can data sets be compared with confidence. Note that the comparability of asbestos is somewhat limited because the accuracy of analysis cannot be easily assessed.

Completeness is defined as the percentage of acceptable sample results compared with the total number of sample results. Completeness equals the total number of sample results for each fraction minus the total number of rejected sample results divided by the total number of sample results multiplied by 100. Percent completeness (%C) is calculated using the following equation:

%C = (T – R)/T × 100,

where:

T = total number of sample results and

R = total number of rejected sample results.

Sensitivity relates to the ability of an analysis method to identify positive results. Specifically for asbestos analysis, sensitivity is measured using a construct called “analytical sensitivity”. This is the calculated concentration of airborne asbestos structures, equivalent to counting one asbestos structure in the analysis (Chatfield, 1995). Analytical sensitivity (S) is a function of the volume of air sampled, the active area of the collection filter and the area of TEM grid that structures are counted (as defined below by Chatfield, 1995):

S = Af / (k*Ag*V)

where Af is the active area of the sample collection filter, Ag is the mean area of grid openings examined, k is the number of grid openings examined and V is the volume of air sampled.

The purpose of the analytical sensitivity is to try to encompass the range of asbestos concentrations that are of concern for asbestos related risk assessment. Berman and Kolk (2000) suggest that an analytical sensitivity of 3 x 106 S/gPM10 will encompass most of these concentrations and is adequate for most studies where protocol amphibole structures are suspected. However, they also suggest that a sensitivity of 5 x 107 S/gPM10 may be sufficient in cases where only chrysotile structures are suspected due to their lower potency compared to amphibole structures.

## Basis for Qualifying Data

Field Duplicates: Duplicate samples are qualified with a J if the RPD between the sample and its duplicate was above 50%.

Blanks: Per EMSL Elutriator SOP (rev. 2.1, June 2010), the following blanks are analyzed:

- Filter lot blanks: 2 per lot of 50 filters, analyzed prior to sampling; lot rejected if background contamination is > 0.2 fiber/mm2;

- Field blanks: determined by client;

- Lab blanks: 1) filter to evaluate elutriator prep room air, 2) filter near elutriator sampling ports (always collected, only analyzed if there is a question of contamination); blanks considered contaminated if >10 structures/mm2;

- Method blank: analyzed 1 every 20 samples (this can take months before 20 samples are analyzed due to the time-intensive elutriator method), washed play sand used to assess tumbler and elutriator, should not exceed 0.2 structures/mm2;

- Equipment blanks: similar to the method blank, except sand is not used (only air); interchangeable with method blank;

- Conditioning filters: collected at the beginning of every run; not required unless there is case of contamination since these filters can help with troubleshooting.

# Asbestos via EPA Method 540-R-97-028 with Berman and Kolk (2000) Modifications  (soils)

Analytical sensitivities for all samples were around 3 x 106 S/gPM10. No qualifications were made based on QC issues, as described below.

## Quality Control Results

### Blank Samples

The following blank information was obtained from EMSL or confirmed via phone with Robyn Denton, EMSL Special Projects Manager:

- Filter lot blanks: running average through 12/4/2008 shows no detects;

- Field blanks: no field blank was included with this data set;

- Lab blanks: no issues have been reported;

- Method blank: no detects thus far;

- Conditioning filters: only used for troubleshooting.

No blank issues were reported; therefore no data was qualified due to blank issues.

### Duplicate Sample Results

Field duplicate results for samples were within the control limits of 50 RPD (established by Berman and Kolk, 2000), except for the following samples: TSB-CR-01-0 / TSB-CR-01-0-FD and TSB-GR-01-0 / TSB-GR-01-0-FD. The results for these samples were J qualified in the EDD.

## Unaddressed Issues

This section details issues that need to be addressed in laboratory reports, Table 4 and Table D10.

Table 4 of the HRA has the following issues:

- Sample ID TSB FJ-05 is listed 3 times as a Sample ID in Table 4 and the Sample Name does not match the ID for these 3 instances. According to Table D10, TSB FJ-05 should be removed. It appears that the Sample IDs should be changed to match the Sample Name in the corresponding column (i.e., P4-PF-1-1-0.0-FD, P4-PH-1-1-0.0, TSB-FJ-05-0-FD).  Similarly, TSB-FR-02 should be removed from the Sample ID column and be replaced with Q3-PF-3-1-0.0.

- The number of long chrysotile fibers for TSB-FJ-10-0 (lab ID 040728237-0032) should be changed from 3 to 2, based on a review of the bench data sheet. The laboratory was counting fibers with widths < 0.5 µm vs. the guidance value of < 0.4 µm. Note that some of these lab reports are rather old and the asbestos guidance has evolved over time. This issue seems to be isolated to SDG 040728237.

Table D10 of the HRA has the following issues:

- Most of the sample dates did not match the sample CoC or EMSL records. The sample dates have been corrected to match the CoC or EMSL records; these are denoted in red in the EDD.

- Table D10 was missing the following samples that were found in Table 4: TSB-HJ-09-NE, TSB-GJ-02-0, TSB-GJ-03-0, TSB-GJ-05-0, TSB-GJ-07-0 and TSB-GR-02-0. These samples were added to the EDD (highlighted in yellow) and sample information was transcribed from laboratory reports.

- There a numerous samples that have incorrect structure counts; these have been corrected in red in the EDD. Note in some instances this was due to the laboratory using screening dimensions different from the risk guidance, specifically regarding structure width. See Table 4 TSB-FJ-10-0 issues above for details.

- The analytical sensitivity for samples P3-PF-1-1-0.0 and P3-PF-2-1-0.0 did not match the laboratory report; these have been corrected in red in the EDD.

- Although the excavated samples (in orange) were not reviewed for this DVSR due to exclusion from the risk calculations, most of the samples were found to have the incorrect sample date. These samples have been excluded from the EDD deliverable, but it is recommended that the sample information and laboratory reports be reviewed for these samples to verify the correct information is being reported in Table D10.

Laboratory reports or supporting information have the following issues:

- Most of the CoC records provided are incomplete with regards to dates and signatures. For example, the CoC associated with SDG 091003471 has only one signature, so it is not clear who relinquished and who received the samples. Similar issues were observed for SDG 040728237.

- The full sample name is not listed on the Bench Sheet Data for samples TSB-CJ-01-0 (lab ID 040728237-0001) and TSB-CJ-02-0 (lab ID 040728237-0002).

- For sample TSB-CJ-07-0 (lab ID 040728237-0007), the Bench Sheet Data (BSD) has discrepancies that need to be clarified; the structure counts cannot be verified. The bottom of pg. 4 of the BSD has a 10 x 0.15 µm structure with “NAM (amphibole)” identification. The NAM mineral class is checked, but it is not clear what the structure really is. If this is a non-regulated amphibole, then the BSD should clarify this. If this structure is a NAM or non-regulated amphibole, the structure counts will be 0 for this sample. Note Table D10 has values above 0 for this sample.

- Samples in SDG 091003269 have two reports (initial and amended). The amended report appears to introduce new issues vs. resolving the original ones for samples. Due to the number of issues found in both reports, it is not clear what to verify data against. The laboratory reports for all of these samples should be verified for correctness with EMSL. Some of the issues found in the file 091003269_SDG_Final_Report_TRNX26.pdf were: 1) 091003269-0004 and 091003269-0007 have the same sample name (P3-PF-2-1-0.0) and 2) the CoC has P4-PF-1-1-0.0 listed twice. Some of the issues found in the file 091003269_SDG_Final_Report_TRNX26_Amended_052010.pdf were: 1) 091003269-0002 (originally P3-PF-1-1-0.0) was changed to Q3-PF-1-1-0.0; 2) 091003269-0003 (originally P4-PH-1-1-0.0) was changed to Q3-PF-1-1-0.0-FD; and 3) the sample counts and/or analytical sensitivities were not matching up with Table D10. Due to the numerous issues found in both files, it is not clear what is correct laboratory report for any of the samples in this SDG. These samples and counts could not be verified. It is recommended that EMSL issue a letter indicating the correct lab ID with the correct customer ID, including corrected CoC documentation. Additionally, there should be an explanation as to why the samples are mixed up and changed between reports to understand the reasoning for some of the changes. There is some email correspondence in the amended files, but the attachments are excluded making it difficult to interpret the issues and how they should be resolved.

- For sample TSB-FR-04-0-FD (lab ID 040728237-0052), the BSD was difficult to read and the structure counts could not be confirmed. It appeared there were 4 long and 4 short chrysotile vs. the 4 long and 5 short chrysotile listed in the laboratory report. Due to the inability to confirm counts, this report should be verified with the laboratory. Additionally, this sample could not be found on the CoC provided for this SDG 040728237.

- The BSD for sample TSB-HJ-08-0 (lab ID 040801461-0020) could not be read; therefore counts could not be confirmed. Note that the counts in Table D10 are not in agreement with each other (e.g., 1 long chrysotile is listed, but there is a 0 for long asbestos structures). The counts for this sample have been corrected to be consistent, but have not been verified against the BSD due to readability issues. The sample date has also been corrected to match EMSL records.

- Lab IDs 040817194-0001 and 040817194-0002 are labeled as TSB-CJ-12-0 and TSB-CJ-13-0, respectively, in the Excel lab report files. It is believed that these samples are actually TSB-HJ-12-0 and TSB-HJ-13-0; this should be verified and corrected as necessary.

- The Excel lab report for sample TSB-HR-01-0 (lab ID 040801461-0005) indicates that there is 1 long chrysotile structure. The BSD does not agree with this, the count should be 0. The Excel report should be corrected.

- The BSDs for samples TSB-HR-08-0 (lab ID 040801461-0018) and TSB-GJ-05-0 (040728237-0042) are not legible. The counts could not be confirmed for these samples.

## Summary

Four samples were qualified due to QC issues, as discussed above. Additionally, there are some unaddressed issues discussed in Section 2.2. Due to the number of issues listed in Section 2.2, it is recommended that the issues be addressed by NERT and a follow-up review be completed by NDEP before official approval of this DVSR.

Beyond the qualifications and unaddressed issues, the data that could be validated are considered acceptable and no data are rejected. The EDD details the verified (or corrected) structure counts and qualifiers (if applicable); this should be used to report asbestos results for these samples.

# PARCCS

Precision: Assessments were discussed above and the precision the data are considered acceptable with the included data qualifiers (where applied).

Accuracy: As discussed above, accuracy is not easily assessed; however, EMSL records indicate the data should be accurate within their limitations.

Representativeness: No significant blank contamination has been found in laboratory samples and the representativeness of the project data is considered acceptable.

Comparability: The laboratory used standard analytical methods for the analyses. No information was provided that would conflict with the comparability of the results; therefore, the overall comparability is considered acceptable.

Completeness: No results were rejected based on this data validation. The completeness level cannot be assessed at this time until the issues in Section 2.2 are addressed.

Sensitivity: The analytical sensitivity for samples validated was around 3 x 106 S/gPM10, which is acceptable for risk assessment. Note that not all samples were validated due to the issues mentioned above in Section 2.2.

# References

Berman DW and Kolk A, (2000). Draft Modified Elutriator Method for the Determination of Asbestos in Soils and Bulk Material, Revision 1. Submitted to the U.S. Environmental Protection Agency, Region 8, May 23, 2000, 84 pp.

Chatfield EJ, (1995). Ambient Air: Determination of Asbestos Fibers, Direct Transfer Transmission Electron Microscopy Procedure. Submitted to the International Standards Organization: ISO 10312:1995(E).

EMSL Elutriator SOP, 2010. Superfund Method for the Determination of Releasable Asbestos in Soils and Bulk Materials with Berman and Kolk Modifications (May 2000). Revision 2.1, June, 18.

NDEP, 2012. Data Validation Guidance for Asbestos Data in Soils for the Basic Management Incorporated (BMI) Complex and Common Areas, June.

NERT, 2013. Post‐Remediation Screening Health Risk Assessment Report for Parcels C, D, F, G and H, Revision 2, Nevada Environmental Response Trust Site, Henderson, Nevada, June.